DOTIFS Fibre Optics Grid Positioning System

PI Stage Controller Software — Development Report

Developed by: Vishal Jain

Date: April 2026

# Table of Contents

1. Project Overview

2. System Architecture

3. Key Features Developed

3.1 GCS Serial Terminal

3.2 Live Dashboard

3.3 Grid Scan System

3.4 Set Your Origin (Manual Stage Positioning)

3.5 Fine Adjust (Nudge Controls)

4. Bug Fixes & Improvements

5. Technical Details

6. Development Timeline

7. Summary

# 1. Project Overview

The DOTIFS (Devasthal Optical Telescope Integral Field Spectrograph) project requires precise placement of optical fibres into a microlens array (MLA) grid. Each fibre must be positioned with micrometer-level accuracy to ensure proper light coupling and spectral quality.

This software provides a web-based controller interface for PI (Physik Instrumente) precision stages, enabling operators to navigate a 12x12 hex-staggered fibre grid and precisely position each fibre using microscope-guided alignment. The application communicates with PI controllers via the Web Serial API using the GCS (General Command Set) protocol.

The system controls two PI linear stages (X and Y axes) in a daisy-chain configuration, each with 1 micrometer resolution. The MLA grid contains 144 fibre positions arranged in a hexagonal pattern with approximately 75 um column pitch and 65 um row pitch.

# 2. System Architecture

The application is built as a single-page HTML application with embedded CSS and JavaScript. It runs directly in Chrome/Edge browser using the Web Serial API for hardware communication.

| Component | Details |
| --- | --- |
| Frontend | Single HTML file with inline CSS/JS |
| Communication | Web Serial API (Chrome/Edge) |
| Protocol | PI GCS (General Command Set) |
| Controllers | PI daisy-chained controllers (ID 1-10) |
| Stage Resolution | 1 micrometer (0.001 mm) |
| Grid Size | 12x12 = 144 fibre positions |

# 3. Key Features Developed

## 3.1 GCS Serial Terminal

A full-featured serial terminal for direct communication with PI controllers:

Connect/disconnect with configurable baud rate (9600 to 115200)

Send any GCS command with controller ID prefix

Real-time response display with color-coded sent/received/error messages

Command history with up/down arrow navigation

Quick-access buttons for common commands: *IDN?, ERR?, SVO ON/OFF, FRF, POS?, ONT?, VEL, MOV, MVR

Command preview showing exact bytes that will be sent

Daisy-chain scanner to auto-detect connected controllers

Emergency stop (#24 / Ctrl+X) for all axes

## 3.2 Live Dashboard

Real-time monitoring of all connected PI controllers:

Auto-scan controllers by ID range

Live polling of POS?, SVO?, ONT?, ERR?, VEL? for each controller

Configurable auto-refresh interval (500ms to 5s)

Smart motion status: shows "servo off" instead of false "moving" when servo is disabled

Quick actions: SVO ON/OFF (all axes 1 & 2), FRF Home, GOH, INI Setup, Emergency Stop

INI (Initialize) button for error recovery on stuck stages

Dashboard command log showing all commands sent

Motion history with timestamps, coordinates, and fibre status

## 3.3 Grid Scan System

The core feature for DOTIFS fibre placement. Manages a 12x12 hex-staggered grid matching the physical MLA fibre array:

Canvas-based hex-staggered grid visualization (even rows offset right by half-pitch)

Load fibre coordinates from .txt file (col row X Y format, values in micrometers)

Automatic micrometer-to-millimeter conversion (file coords / 1000) for PI controller commands

All movements are MVR (relative) from user-set origin - never absolute to file coordinates

Movement sequence: MOV to origin -> wait on-target -> MVR by file offset

Confirm dialog shows exact GCS commands before executing any move

Command log panel on right side showing all sent commands with timestamps

Point states: Unvisited (grey), Current (amber), Fibre inserted (green tick), No fibre (dark), Skipped/Deleted (red X)

Skip/Delete points - auto-scan jumps over deleted points, with restore option

Serpentine traversal (boustrophedon) - alternating direction per row/column to minimize travel

Save/Load progress as JSON, auto-save to localStorage

Password-protected access to grid scan tab

Origin display locked in status strip with lock icon

"Go to Origin" button to return both axes to set origin

## 3.4 Set Your Origin (Manual Stage Positioning using JOG)

Modal dialog for precisely setting the grid origin (position of fibre hole 0,0). The operator manually moves (jogs) the stages in small increments using arrow buttons to align the fibre insertion point under the microscope, then locks that position as the reference origin for all grid movements:

Live position readout for both X and Y axes (mm)

Step size presets: 0.1, 1, 10, 100, 1000 micrometer with custom input

Manual move buttons for X axis: -100x, -1x step, +1x step, +100x

Manual move buttons for Y axis: same layout

FRF Both -> Lock as Origin: homes both axes and auto-sets as grid origin

FRF Both (no lock): homes axes without changing origin

Velocity control: set same velocity on both X and Y axes (mm/s)

Read velocity: query current velocity of both axes

Lock as Origin: saves current position as grid (0,0) reference

Unlock / Clear: removes origin lock

Origin persisted in localStorage across sessions

Dashboard auto-polling paused while jog panel is open (prevents scanQuery conflicts)

## 3.5 Fine Adjust (Nudge Controls)

Arrow-pad controls on the grid scan right panel for fine fibre alignment while viewing through microscope:

Up/Down arrows for Y axis nudge, Left/Right for X axis nudge

Uses MVR (relative move) for precise adjustments

Step presets: 0.1 um, 1 um, 5 um, 10 um, 50 um

Custom step input in mm (default 0.001 mm = 1 um = stage resolution)

Shows last sent command for verification

Step label auto-converts: nm / um / mm display

# 4. Bug Fixes & Improvements

| Issue | Root Cause | Fix |
| --- | --- | --- |
| Dashboard shows "moving" when stage is idle | ONT? returns 0 when servo is off, interpreted as moving | Query SVO? before ONT?; show "servo off" instead of "moving" when SVO=0 |
| SVO OFF does not turn off all devices | Only sent to axis 1, not axis 2 | Loop over axis 1 and axis 2 for each controller |
| Stage error never clears | No INI command available in UI | Added INI button + "Recover Axis" sequence (SVO OFF -> INI -> SVO ON) |
| Jog panel X/Y position read fails | Overlapping scanQuery calls; dashboard polling conflicts | Added jogPolling guard, increased timeouts, pause dashboard during jog |
| Grid coordinates sent as mm instead of um | File values in micrometers sent directly as mm (1000x too far) | Added gsUnitToMM() conversion, unit selector defaulting to um |
| Grid stagger on wrong rows | Odd rows staggered instead of even rows | Swapped row%2===1 to row%2===0 in gsCellXY() |
| MOV to absolute file coordinates | Stage moved to raw file values instead of relative from origin | Changed to MVR-only mode: MOV to origin, then MVR by offset/1000 |

# 5. Technical Details

## GCS Commands Used

| Command | Purpose |
| --- | --- |
| *IDN? | Identify controller |
| ERR? | Read error register (0 = OK) |
| SVO {axis} 1/0 | Enable/disable servo closed-loop |
| FRF {axis} | Reference axis using home switch |
| POS? {axis} | Query current position |
| ONT? {axis} | Query on-target status (1 = arrived) |
| VEL {axis} {val} | Set velocity in mm/s |
| MOV {axis} {pos} | Absolute move to position (mm) |
| MVR {axis} {dist} | Relative move by distance (mm) |
| INI {axis} | Initialize/reset axis (clears errors) |
| HLT {axis} | Halt axis (decelerate) |
| 0x18 (Ctrl+X) | Emergency stop all axes (#24) |

## Movement Logic

All grid movements follow this sequence:

User sets origin via "Set Your Origin" (jog to position or FRF Both)

Origin position (e.g., X=100.000, Y=100.000 mm) is locked

User clicks grid point (e.g., col=0, row=1 with file coords X=37.895, Y=64.974 um)

Confirm dialog shows exact commands to be sent

Step 1: MOV both axes to origin (absolute move to locked position)

Step 2: Wait for both axes on-target (ONT? polling)

Step 3: MVR X by 37.895/1000 = 0.037895 mm (relative from origin)

Step 4: MVR Y by 64.974/1000 = 0.064974 mm (relative from origin)

Step 5: Wait for on-target, show fibre insertion panel

## Coordinate File Format

The MLA coordinate file (MLA_2_I1_3_rot_cropped.txt) contains 144 lines in the format:

col  row  X_um  Y_um

Example: "0 1 37.894991598762175 64.97365764186839"

Grid properties: 12 columns x 12 rows, column pitch ~75 um, row pitch ~65 um, hex stagger ~38 um on even rows. Total grid extent: ~829 um x 715 um (< 1 mm x 1 mm).

# 6. Development Timeline

All development completed on 7-8 April 2026:

| Commit | Description |
| --- | --- |
| 994c25e | Initial project setup - HTML pages, images, .gitignore |
| 65ef4db | Fix dashboard false-moving, SVO OFF all axes, INI error recovery, hex grid |
| 2542297 | Add FRF Both XY button and micrometer-to-millimeter unit conversion |
| 34c3d26 | Swap grid stagger: even rows shifted right to match physical MLA |
| 3ae7874 | Fix jog panel position read (X/Y) and auto-set origin after FRF |
| 5995841 | Add nudge controls for fine fibre adjustment (arrow pad with step presets) |
| 2587b62 | Add MLA fibre coordinate file (12x12 hex grid, micrometers) |
| 8106b04 | Add skip/delete points, MVR mode, move-to-origin button |
| 10bedc0 | Remove Gen Hex Grid (unused with .txt file loading) |
| 7ea9e64 | Show command preview in confirm dialog, add grid command log |
| 1a04e33 | Rename Jog to "Set Your Origin", MVR-only mode, remove MOV toggle |
| dc83345 | Add FRF Both & velocity controls to Set Your Origin modal |

# 7. Summary

The DOTIFS Fibre Optics Grid Positioning System provides a complete, browser-based solution for precise fibre placement into the microlens array. The software enables operators to:

Control PI precision stages with 1 micrometer resolution via Web Serial

Navigate a 144-point hex-staggered grid matching the physical MLA layout

Set a reference origin and move to each fibre position using relative (MVR) commands

Fine-adjust fibre alignment using nudge controls while viewing through a microscope

Track progress with visual grid states, skip/delete points, and auto-save

Monitor all controllers in real-time with error detection and recovery (INI)

Verify every command before execution via the command preview and log

Report prepared by: Vishal Jain

Project: DOTIFS - Devasthal Optical Telescope Integral Field Spectrograph

Repository: https://github.com/allwhc/dotifs_guide_fibre

Date: April 2026